Drill Hole (3x) (Align bottom of sheet with bottom of tube) 24th and Elm Street  Dubuque, IA 52001 Ph: 800-553-4840 Fax: 563-583-5028 Email: custserv@morbros.com 539B--0405 PP, Rev A INSTALLATION INSTRUCTIONS FOR 4” FIG. 539B DIFFUSER - Check drop tube: diffuser is made for a 4” drop tube with an inside diameter between 3-23/32” and 3-49/64”. - Cut drop tube square so that the bottom of the drop tube is 6” from the bottom of the tank. - Place this instruction onto the tube so the bottom edge of the sheet is flush with the bottom edge of the tube. - Punch the locations of the holes using the template below and then drill through the tube with a 5/16” drill. - Place diffuser on tube, align holes and secure with enclosed countersunk screws. Tighten screws until they are flush with the outer diameter of the drop tube. WARNING: If drop tube has a built in overfill valve, make sure the diffuser through hole is properly aligned with the open side of the overfill valve.

Drill Hole (3x) (Align bottom of sheet with bottom of tube) 24th and Elm Street  Dubuque, IA 52001 Ph: 800-553-4840 Fax: 563-583-5028 Email: custserv@morbros.com 539B--0405 PP, Rev A INSTALLATION INSTRUCTIONS FOR 4” FIG. 539B DIFFUSER 1. Check drop tube: diffuser is made for a 4” drop tube with an inside diameter between 3-23/32” and 3-49/64”. - Cut drop tube square so that the bottom of the drop tube is 6” from the bottom of the tank. - Place this instruction onto the tube so the bottom edge of the sheet is flush with the bottom edge of the tube. - Punch the locations of the holes using the template below and then drill through the tube with a 5/16” drill. - Place diffuser on tube, align holes and secure with enclosed countersunk screws. Tighten screws until they are flush with the outer diameter of the drop tube. WARNING: If drop tube has a built in overfill valve, make sure the diffuser through hole is properly aligned with the open side of the overfill valve.

Drill Hole (3x) (Align bottom of sheet with bottom of tube) 24th and Elm Street  Dubuque, IA 52001 Ph: 800-553-4840 Fax: 563-583-5028 Email: custserv@morbros.com 539B--0405 PP, Rev A INSTALLATION INSTRUCTIONS FOR 4” FIG. 539B DIFFUSER - Check drop tube: diffuser is made for a 4” drop tube with an inside diameter between 3-23/32” and 3-49/64”. - Cut drop tube square so that the bottom of the drop tube is 6” from the bottom of the tank. - Place this instruction onto the tube so the bottom edge of the sheet is flush with the bottom edge of the tube. - Punch the locations of the holes using the template below and then drill through the tube with a 5/16” drill. - Place diffuser on tube, align holes and secure with enclosed countersunk screws. Tighten screws until they are flush with the outer diameter of the drop tube. WARNING: If drop tube has a built in overfill valve, make sure the diffuser through hole is properly aligned with the open side of the overfill valve.